# TeleLogs RCA Feature Engineering Master Documentation

Purpose: This document summarizes the 50 engineered features used in the telecom RCA modeling dataset, including feature type, meaning, raw variables, and calculation/formula.

Notes: Throughput low-period features use the TeleLogs symptom threshold of throughput < 600 Mbps. Distance is computed using the Haversine formula from user GPS coordinates and serving-cell engineering coordinates. Effective tilt uses Mechanical Downtilt + cleaned Digital Tilt, where Digital Tilt = 255 is treated as 6 degrees.

| Feature | Type | Meaning | Raw Variable(s) | Calculation / Formula |
| --- | --- | --- | --- | --- |
| avg_throughput | Numeric | Average downlink throughput across the drive-test scenario. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | mean(throughput) |
| min_throughput | Numeric | Lowest downlink throughput observed; captures worst-case performance. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | min(throughput) |
| std_throughput | Numeric | Variation/instability of throughput during the scenario. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | standard deviation(throughput) |
| low_tp_count | Numeric | Number of timestamps where throughput dropped below the 600 Mbps symptom threshold. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | count(throughput < 600) |
| low_tp_ratio | Numeric | Proportion of timestamps with low throughput. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | mean(throughput < 600) or low_tp_count / total rows |
| avg_throughput_low_tp | Numeric | Average throughput during only low-throughput periods. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | mean(throughput where throughput < 600) |
| consecutive_low_tp_max | Numeric | Longest consecutive streak of timestamps with low throughput. | user_df: 5G KPI PCell Layer2 MAC DL Throughput [Mbps] | maximum consecutive run length where throughput < 600 |
| max_speed | Numeric | Maximum vehicle speed during the scenario. | user_df: GPS Speed (km/h) | max(GPS Speed) |
| pct_speed_above_40 | Numeric | Proportion of timestamps where vehicle speed exceeded 40 km/h; relevant to C7. | user_df: GPS Speed (km/h) | mean(GPS Speed > 40) |
| avg_speed_low_tp | Numeric | Average vehicle speed during low-throughput periods. | user_df: GPS Speed (km/h), throughput | mean(GPS Speed where throughput < 600) |
| max_speed_low_tp | Numeric | Maximum vehicle speed during low-throughput periods. | user_df: GPS Speed (km/h), throughput | max(GPS Speed where throughput < 600) |
| num_unique_serving_pci | Numeric | Number of unique serving PCIs observed; reflects serving-cell changes. | user_df: 5G KPI PCell RF Serving PCI | nunique(serving PCI) |
| handover_count | Numeric | Number of serving PCI switches; proxy for handovers. | user_df: 5G KPI PCell RF Serving PCI | count(serving PCI changes between consecutive rows) |
| handover_rate | Numeric | Rate of serving-cell changes over the scenario. | user_df: 5G KPI PCell RF Serving PCI | handover_count / (number of rows - 1) |
| handover_count_low_tp | Numeric | Number of serving PCI switches during low-throughput periods. | user_df: serving PCI, throughput | count(serving PCI changes within rows where throughput < 600) |
| avg_rsrp | Numeric | Average serving-cell RSRP; captures overall serving signal strength. | user_df: 5G KPI PCell RF Serving SS-RSRP [dBm] | mean(serving RSRP) |
| min_rsrp | Numeric | Weakest serving-cell RSRP observed; captures worst-case signal strength. | user_df: 5G KPI PCell RF Serving SS-RSRP [dBm] | min(serving RSRP) |
| avg_rsrp_low_tp | Numeric | Average serving-cell RSRP during low-throughput periods. | user_df: serving RSRP, throughput | mean(serving RSRP where throughput < 600) |
| avg_sinr | Numeric | Average serving-cell SINR; captures signal quality/interference conditions. | user_df: 5G KPI PCell RF Serving SS-SINR [dB] | mean(serving SINR) |
| min_sinr | Numeric | Lowest serving-cell SINR observed; captures worst-case signal quality. | user_df: 5G KPI PCell RF Serving SS-SINR [dB] | min(serving SINR) |
| avg_sinr_low_tp | Numeric | Average SINR during low-throughput periods. | user_df: serving SINR, throughput | mean(serving SINR where throughput < 600) |
| std_sinr | Numeric | Variation/instability in serving-cell SINR. | user_df: 5G KPI PCell RF Serving SS-SINR [dB] | standard deviation(serving SINR) |
| pct_sinr_below_5 | Numeric | Proportion of timestamps with poor SINR below 5 dB. | user_df: 5G KPI PCell RF Serving SS-SINR [dB] | mean(serving SINR < 5) |
| min_sinr_low_tp | Numeric | Lowest SINR during low-throughput periods. | user_df: serving SINR, throughput | min(serving SINR where throughput < 600) |
| avg_rb | Numeric | Average number of scheduled downlink resource blocks. | user_df: 5G KPI PCell Layer1 DL RB Num (Including 0) | mean(RB Num) |
| min_rb | Numeric | Minimum scheduled resource blocks; captures worst-case capacity allocation. | user_df: 5G KPI PCell Layer1 DL RB Num (Including 0) | min(RB Num) |
| avg_rb_low_tp | Numeric | Average scheduled RBs during low-throughput periods. | user_df: RB Num, throughput | mean(RB Num where throughput < 600) |
| min_rb_low_tp | Numeric | Minimum scheduled RBs during low-throughput periods. | user_df: RB Num, throughput | min(RB Num where throughput < 600) |
| pct_rb_below_160 | Numeric | Proportion of timestamps with RB allocation below 160; relevant to C8. | user_df: 5G KPI PCell Layer1 DL RB Num (Including 0) | mean(RB Num < 160) |
| avg_neighbor_advantage | Numeric | Average advantage of strongest neighbor signal relative to serving-cell RSRP. Less negative or positive values suggest stronger neighbor influence. | user_df: serving RSRP, Top 1-5 neighbor filtered Tx BRSRP [dBm] | For each row: max(neighbor BRSRP) - serving RSRP; then mean across rows |
| max_neighbor_advantage | Numeric | Maximum observed neighbor advantage over serving cell. | user_df: serving RSRP, Top 1-5 neighbor filtered Tx BRSRP [dBm] | max(max(neighbor BRSRP) - serving RSRP) |
| neighbor_stronger_ratio | Numeric | Proportion of timestamps where strongest neighbor is stronger than serving cell. | user_df: serving RSRP, Top 1-5 neighbor filtered Tx BRSRP [dBm] | mean(max(neighbor BRSRP) > serving RSRP) |
| avg_close_neighbor_count | Numeric | Average number of neighbors within 3 dB of serving-cell RSRP; proxy for close competing coverage. | user_df: serving RSRP, Top 1-5 neighbor filtered Tx BRSRP [dBm] | For each row: count(¦neighbor BRSRP - serving RSRP¦ <= 3); then mean |
| overlap_ratio | Numeric | Proportion of timestamps with at least one close neighbor; proxy for overlapping coverage. | user_df: serving RSRP, Top 1-5 neighbor filtered Tx BRSRP [dBm] | mean(close_neighbor_count > 0) |
| same_mod30_exists | Numeric / Binary | Whether any neighbor PCI has the same PCI mod 30 as the serving PCI; relevant to C6. | user_df: serving PCI, Top 1-5 neighbor PCI | 1 if any neighbor PCI % 30 == serving PCI % 30, else 0 |
| same_mod30_count | Numeric | Total count of same mod 30 conflicts across timestamps/neighbors. | user_df: serving PCI, Top 1-5 neighbor PCI | sum(count(neighbor PCI % 30 == serving PCI % 30) per row) |
| same_mod30_ratio | Numeric | Proportion of timestamps with at least one same mod 30 PCI conflict. | user_df: serving PCI, Top 1-5 neighbor PCI | mean(any neighbor PCI % 30 == serving PCI % 30) |
| avg_serving_tilt | Numeric | Average effective tilt of the serving cell during the scenario. | user_df: serving PCI; eng_df: Mechanical Downtilt, Digital Tilt, PCI | effective_tilt = Mechanical Downtilt + digital_tilt_clean; then mean for serving PCI rows |
| max_serving_tilt | Numeric | Maximum effective tilt among serving-cell observations. | user_df: serving PCI; eng_df: Mechanical Downtilt, Digital Tilt, PCI | max(effective_tilt for serving PCI rows) |
| avg_serving_beamwidth | Numeric | Average vertical beamwidth of serving cell based on beam scenario mapping. | user_df: serving PCI; eng_df: Beam Scenario, PCI | Map Beam Scenario to 6, 12, or 25 degrees; then mean for serving PCI rows |
| avg_serving_height | Numeric | Average antenna height of the serving cell. | user_df: serving PCI; eng_df: Height, PCI | mean(Height for serving PCI rows) |
| avg_serving_distance_km | Numeric | Average geographic distance between UE and serving cell during the drive test. | user_df: Longitude, Latitude, serving PCI; eng_df: Longitude, Latitude, PCI | For each row: haversine(user GPS, serving-cell GPS); then mean distance in km |
| max_serving_distance_km | Numeric | Maximum geographic distance between UE and serving cell. | user_df: Longitude, Latitude, serving PCI; eng_df: Longitude, Latitude, PCI | max(haversine(user GPS, serving-cell GPS)) |
| pct_distance_gt_1km | Numeric | Proportion of serving-cell observations where UE is more than 1 km from serving cell; relevant to C2. | user_df: Longitude, Latitude, serving PCI; eng_df: Longitude, Latitude, PCI | mean(serving distance > 1 km) |
| avg_distance_low_tp | Numeric | Average serving-cell distance during low-throughput periods. | user_df: Longitude, Latitude, serving PCI, throughput; eng_df: Longitude, Latitude, PCI | mean(serving distance where throughput < 600) |
| txrx_mode | Categorical | Most common Tx/Rx antenna configuration among engineering parameters in the scenario. | eng_df: TxRx Mode | mode(TxRx Mode); Unknown if missing |
| antenna_model | Categorical | Most common antenna model among engineering parameters in the scenario. | eng_df: Antenna Model | mode(Antenna Model); Unknown if missing |
| throughput_per_sinr | Numeric | Throughput efficiency relative to signal quality; higher values may indicate high throughput despite given SINR. | Engineered from avg_throughput, avg_sinr | avg_throughput / (avg_sinr + 1) |
| tilt_rsrp_interaction | Numeric | Interaction capturing high serving tilt combined with weak signal; designed to strengthen C1 separation. | Engineered from avg_serving_tilt, min_rsrp | avg_serving_tilt * abs(min_rsrp). abs() is used because RSRP is negative and larger absolute value means weaker signal. |
| neighbor_competition | Numeric | Interaction capturing neighbor influence under overlapping coverage; designed to help separate C3 and C4. | Engineered from avg_neighbor_advantage, overlap_ratio | avg_neighbor_advantage * overlap_ratio |